# Embedded Systems - Key Concepts Notes - ENG. ADEL SHATA

ARM Cortex-M3 | STM32F103 | Bare-Metal Programming

# Q1: Why is the main function's address in FLASH (0x080000B4) and not in SRAM?

The main() function is CODE (instructions), not data. Code lives in FLASH, not SRAM. The CPU fetches instructions from FLASH and executes them there. It does not copy code to SRAM.

## FLASH vs SRAM Responsibilities

| FLASH (permanent) | SRAM (volatile, fast) |
| --- | --- |
| Interrupt Vector Table | Stack (grows downward) |
| All code / functions (main, Reset_Handler, etc.) | Local variables (automatic) |
| Read-only data (.rodata) | Return addresses |
| const global variables | Function parameters |
|  | Global variables (.data) |
|  | Uninitialized globals (.bss) |

# Q2: If main is in FLASH, what is the point of the stack?

The stack is for DATA, not code. When main() runs from FLASH, it uses the stack in SRAM to store its local variables, return addresses, and function parameters.

## What the stack does

| Stack Item | Where it lives | Purpose |
| --- | --- | --- |
| Local variables (e.g., uint16 i) | SRAM (stack) | Temporary data used by functions |
| Return addresses | SRAM (stack) | Where to return after a function call |
| Function parameters | SRAM (stack) | Arguments passed between functions |
| Compiler temporaries | SRAM (stack) | Intermediate calculation results |

## Concrete Example

int main(void)  // instruction is in FLASH at 0x080000B4
{
    while(1)
    {
        for(uint16 i = 0; i < 10000; i++);  // 'i' is LOCAL -> lives on STACK in SRAM
    }
}

# Q3: When I call a function, does it take place in the stack?

No. The function's CODE stays in FLASH. Only its DATA goes on the stack.

## What happens when main() calls foo()

1. CPU pushes the return address onto the stack (where to come back to).

2. CPU jumps to foo() — the code is still in FLASH.

3. foo()'s local variables (x, y, z) are allocated on the stack in SRAM.

4. foo() executes its instructions from FLASH.

5. When foo() returns, the return address is popped from the stack, and execution goes back to main().

## Key Distinction

| What | Where |
| --- | --- |
| Function instructions (code) | FLASH — never moves |
| Return address | SRAM (stack) — where to come back to |
| Local variables | SRAM (stack) — temporary data |
| Global variables | SRAM (.data/.bss) or FLASH (.rodata) |

# Q4: Is this the normal workflow within all projects?

Yes. This is the standard workflow for virtually all bare-metal and embedded projects. Code lives in FLASH, data lives in SRAM. This is fundamental to Harvard and von Neumann architectures used in microcontrollers.

## Universal Rule Across All Microcontrollers

| Architecture | Code Location | Data Location |
| --- | --- | --- |
| ARM Cortex-M (STM32, etc.) | FLASH | SRAM |
| AVR (Arduino) | FLASH | SRAM |
| PIC | FLASH | RAM |
| MSP430 | FLASH/ROM | RAM |
| RISC-V | FLASH | SRAM |

## Exceptions (rare)

Running from RAM for speed (rare, specialized)

External memory (large arrays, buffers)

Memory-mapped peripherals (registers at fixed addresses)

Summary: For 99% of embedded projects: code in FLASH, data in SRAM.

# Q5: How to run code from RAM for speed?

You don't copy ALL of .text — only the specific functions you want to run from RAM. You need three things: the section attribute on the function, the >ram AT> flash linker directive, and the copy routine in startup.

## Step 1: Mark functions for RAM execution

__attribute__((section(".ramfunc")))
void critical_function(void) {
    // This will run from RAM
}

void normal_function(void) {
    // This stays in FLASH
}

## Step 2: Linker script

SECTIONS
{
    .text :
    {
        *(.vectors*)
        *(.text*)
        *(.rodata*)
    } > flash

    .ramfunc :       /* special section for RAM functions */
    {
        *(.ramfunc*)
    } > ram AT> flash   /* lives in SRAM, loaded from FLASH */

    .data :
    {
        *(.data*)
    } > sram

    .bss :
    {
        *(.bss*)
    } > sram
}

## Step 3: Startup code — copy .ramfunc

// Copy .ramfunc from FLASH to SRAM
src = &_E_text;              // load address in FLASH
dst = &_S_ramfunc;           // destination in SRAM
size = &_E_ramfunc - &_S_ramfunc;
for (uint32 i = 0; i < size; i++)
    *((uint32 *)dst++) = *((uint32 *)src++);

## Why it is faster

| Memory | Access Speed |
| --- | --- |
| FLASH | ~2-4 wait states (slow) |
| SRAM | 0 wait states (fast) |

# Quick Reference Summary

| Concept | Key Point |
| --- | --- |
| Code (instructions) | Always in FLASH. CPU fetches and executes from FLASH. |
| Stack | In SRAM. Used for local variables, return addresses, parameters. |
| Local variables, Const Local variables | Live on the stack in SRAM, even though the code using them is in FLASH. |
| (Initialized)Global variables, Static Vars(global or local) | data in SRAM (copied from FLASH at startup) |
| (Unitialized)Global variables, Static Vars(global or local) | .bss in SRAM (zeroed in Ram within startup’s runstime). |
| Const Global variables, String | .rodata(Flash) |
| Function calls | Code stays in FLASH. Only data (return address, locals) goes on the stack. |
| Running from RAM | Mark functions with __attribute__((section(".ramfunc"))), use >ram AT> flash, copy at startup. |